01 Matlab Code

- Scale Transformation Function

설명 : 스케일 변환 함수

% Scaling transformation function

function y = Scale(x, max_value, min_value)

y = (255/(max_value-min_value)*(x-min_value));

end

- Equalize Transformation Function

설명 : 평활화 변환 함수

%Equalize Transform Fuction, x is histogram value

function y = ETF(x)

m = size(x);

sum = 0;

for i = 1:m

sum = sum+x(i);

y(i,1) = sum;

end

end

- Image Equalizing Fuction

설명 : 이미지가 ETF에 의해 평활화 되도록 도와주는 함수

% x1 is original image and x2 is ETF

function y = EF(x1, x2)

[m, n] = size(x1);

for i = 1:m

for j = 1:n

y(i, j) = x2(x1(i, j)+1);

end

end

- Main Code

%%% Robot Vision%%%

%%% Dept. of Electronic Engineering

%%% 201314651 Lee Wonjai

IM_Pollen = imread('C:\Users\user\OneDrive\¹ÙÅÁ È­¸é\4ÇÐ³â 2ÇÐ±â\·Îº¿ºñÀü\Original Images\dipum_images_ch03\Fig0308(a)(pollen).tif'); % read the targeted image

IM_Moon = imread('C:\Users\user\OneDrive\¹ÙÅÁ È­¸é\4ÇÐ³â 2ÇÐ±â\·Îº¿ºñÀü\Original Images\dipum_images_ch03\Fig0310(a)(Moon Phobos).tif');

P_Pollen = imhist(IM_Pollen);

P_Moon = imhist(IM_Moon);

%Find Max and Min value of Pollen Image

Min1 = min(find(P_Pollen))-1; %also can be done using min and max value of IM_Pollen

Max1 = max(find(P_Pollen))-1; %the reason of -1 is that array in matlab start to calculate the index of array as 1

%Find Max and Min value of Moon Image

Min2 = min(find(P_Moon))-1;

Max2 = max(find(P_Moon))-1;

IMScale_Pollen = Scale(IM_Pollen, Max1, Min1);

IMScale_Moon = Scale(IM_Moon, Max2, Min2);

PScale_Pollen = imhist(IMScale_Pollen);

PScale_Moon = imhist(IMScale_Moon);

[m1, n1] = size(IM_Pollen);

[m2, n2] = size(IM_Moon);

ETF_Pollen = ETF(P_Pollen)./(m1.*n1).*255;

ETF_Moon = ETF(P_Moon)./(m2.*n2).*255;

EF_Pollen = uint8(EF(IM_Pollen, ETF_Pollen));

EF_Moon = uint8(EF(IM_Moon, ETF_Moon));

CEF_Pollen = histeq(IM_Pollen, 256);

CEF_Moon = histeq(IM_Moon, 256);

% Compare the histogram of Original Image and scale transformed Image

figure(1);

subplot(1,2,1);

imhist(IM_Pollen)

title('Original Image of Pollen')

subplot(1,2,2);

imhist(IMScale_Pollen)

title('Scale transformed Image of Pollen')

figure(2);

subplot(1,2,1);

imhist(IM_Moon)

title('Original Image of the Moon')

subplot(1,2,2);

imhist(IMScale_Moon)

title('Scale transformed Image of the Moon')

% Compare the Original Image and scale transformed Image

figure(3);

subplot(1,2,1);

imshow(IM_Pollen)

title('Original Image of Pollen')

subplot(1,2,2);

imshow(IMScale_Pollen)

title('Scale transformed Image of Pollen')

figure(4)

subplot(1,2,1);

imshow(IM_Moon)

title('Original Image of the Moon')

subplot(1,2,2);

imshow(IMScale_Moon)

title('Scale transformed Image of the Moon')

% Compare the histogram of Original Image and equalized Image

figure(5);

subplot(1,2,1);

imhist(IM_Pollen)

title('Original Image of Pollen')

subplot(1,2,2);

imhist(EF_Pollen)

title('Equalized Image of Pollen')

figure(6);

subplot(1,2,1);

imhist(IM_Moon)

title('Original Image of the Moon')

subplot(1,2,2);

imhist(EF_Moon)

title('Equalized Image of the Moon')

% Compare the Original Image and equalized Image

figure(7);

subplot(1,2,1);

imshow(IM_Pollen)

title('Original Image of Pollen')

subplot(1,2,2);

imshow(EF_Pollen)

title('Equalized Image of Pollen')

figure(8);

subplot(1,2,1);

imshow(IM_Moon)

title('Original Image of the Moon')

subplot(1,2,2);

imshow(EF_Moon)

title('Equalized Image of the Moon')

%Compare whether equalization that I made is correct or not

figure(9)

subplot(1,2,1);

imhist(EF_Pollen)

title('Equalization Fuction I made')

subplot(1,2,2);

imhist(CEF_Pollen)

title('Equalization Fuction')

figure(10)

subplot(1,2,1);

imhist(EF_Moon)

title('Equalization Fuction I made')

subplot(1,2,2);

imhist(CEF_Moon)

title('Equalization Fuction')

figure(11)

subplot(1,2,1);

imshow(EF_Pollen)

title('Equalization Fuction I made')

subplot(1,2,2);

imshow(CEF_Pollen)

title('Equalization Fuction')

figure(12)

subplot(1,2,1);

imshow(EF_Moon)

title('Equalization Fuction I made')

subplot(1,2,2);

imshow(CEF_Moon)

title('Equalization Fuction')

02 Result 1

- Histogram of Original and Scaled Image of Pollen

- Histogram of Original and Scaled Image of the Moon

- Original and Scaled Image of Pollen

- Original and Scaled Image of the Moon

- Histogram of Original and Equalized Image of Pollen

- Histogram of Original and Equalized Image of the Moon

- Original and Equalized Image of Pollen

- Original and Equalized Image of the Moon

- Check the Function that I made whether right or not (Pollen)

- Check the Function that I made whether is right or not (Moon)

- Check the Function that I made whether right or not (Pollen)

- Check the Function that I made whether right or not (Moon)

02 심화학습(python 구현)

Result 2

- Original and Scaled Image of Pollen

- Original and Scaled Image of the Moon

- Original and Equalized Image of Pollen

- Original and Equalized Image of Moon

- Histogram of the Originals and It’s Images

- Histogram of Original and Scaled Images

- Histogram of Original and Equalized Images

Conclusion

- Matlab

- Scale 처리의 경우 Pollen은 좋은 이미지가 되었고, 이를 히스토그램에서 확인할 수 있었다. 그러나 Moon은 큰 차이가 없었다. 그 이유는 히스토그램을 보면 기존의 Moon 이미지 픽셀 값 자체가 넓게 분포되어 있기 때문이고, 특히 어두운 값(1에 가까운 값) 밝은 값(255에 가까운 값)에 집중 분포 되어있기에 Scale 처리 효과가 미미하다.

- Equalization은 이미지 전체 밝기를 전반적으로 밝게 해준다. 따라서 Moon의 경우에도 Scale에서 미처 하지 못했던 기능을 수행한다. 다만 전반적 밝기가 밝아져, Contrast가 약해졌다. 그럼에도 불구하고 어두운 부분의 이미지 판별이 필요한 경우가 있어 용이한 사진이다.

- Main Code에서 Scale 범위 찾을 때 find 함수 쓴 이유: 단순히 히스토그램에서 0이 아닌 값의 시작과 끝을 찾는다는 의미를 주기 위해서이다. 그러나 굳이 find 쓸 필요 없이 이미지에 min(min()) 또는 max(max())를 쓰면 컴퓨터적으로 계산이 더 쉽다.

- Equalization할 때에는 Scale과 다르게 함수 식을 세워 대입하는 것이 아니기에 CDF에 이미지 값을 직접 for구문을 써서 대입해준다.

- Python

- 이번 python의 경우 library로 cv2, numpy, matplotlib를 썼다. 특히 matplotlib의 경우 매트랩과 비슷한 개발 환경을 제공해주는 라이브러리이기에 두 영상 간의 변화를 비교하기에 좋다.

- Matplotlib를 이용했기에 plt 함수를 이용하여, Matlab처럼 figure, subplot을 만드는 것이 가능해짐.

- Matplotlib은 주로 데이터를 시각화 할 때 많이 쓰인다.